| Individual Candidate Marking Sheet | WAH Instructor |
| --- | --- |

Candidate Name:_________________

| Skill | Criteria |  | Total |
| --- | --- | --- | --- |
| Carry out a range of techniques | - Labels - Heat or friction damage: glazed appearance - Buckles, D rings for cracks, deformations etc. - Buckle systems working properly - Ultraviolet degradation: some loss of colour - Contamination (e.g. with dirt, grit, sand etc.) - Beginning of breaks in webbing - Damaged stitching & webbing (cuts etc) - Chemical attacks often indicated by flaking - Impact/ shock absorber intact - Lanyard: condition, knots, twisted - Connectors, Fitting - }Hold harness by D-ring & shake out! - }Release chest, leg and/or waist straps - }Slip straps over shoulders - }Locate D-ring in middle of shoulder blades - }Pull leg straps between legs and connect - }Adjust thigh straps by pulling/ releasing end - }If belted connect waist strap after leg straps. - }Connect chest strap buckle and centre - }Tighten to keep shoulder straps taut. - }Tighten straps so the harness fits snugly |  |  |
| Presentation |  |  |  |

Signature_________________________________ Date: